TECHNICAL SPECIFICATION FOR TRAVOLATOR

| Buyer |  |  |  |  |  | Project Name |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contract No. |  |  |  |  |  | Project Code |  |  |  |  |  |  |  |  |  |  |  |
| Installation site |  |  |  |  |  | Quotation No. |  |  |  |  |  |  |  |  |  |  |  |
| Delivery ex works |  |  |  |  |  | Quantity |  |  |  |  |  |  |  |  |  |  |  |
| Ⅰ Main feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Pallet width (mm) |  | 1000 |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  |
| 2 | Horizontal movement (mm) |  | 400 (at top for AET, at both landing for AEF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Vertical rise |  | H= mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Horizontal span |  | L = mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Inclination |  | 10° |  |  |  | 11° |  |  |  |  |  | 12° |  |  |  |  |
| 6 | Speed |  | 0.5m/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Power voltage |  | 380V |  |  |  | 400V |  |  |  |  |  |  | 415V |  |  | V |
| 8 | Lighting voltage |  | 220V |  |  |  | 230V |  |  |  |  |  |  | 240V |  |  | V |
| 9 | Frequency |  | 50HZ |  |  |  |  |  |  |  |  |  |  | 60HZ |  |  |  |
| 10 | Motor |  | kW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Gear box |  | Worm gear |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Arrangement |  | Single |  |  |  |  | Parallel |  |  |  |  |  |  | Continuous |  |  |
|  |  |  | Crisscross |  |  |  |  | According to layout |  |  |  |  |  |  |  |  |  |
| 13 | Treatment of truss |  | Normal painting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Delivery in |  | Whole piece |  |  |  |  | Two pieces |  |  |  |  |  |  | pieces |  |  |
| 15 | Intermediate support |  | No |  |  |  |  | One |  |  |  |  |  |  |  |  |  |
| 16 | Installation |  | Indoor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Standard installation |  | Yes |  |  |  |  | No (If no, please fill 17.1 and17.2) |  |  |  |  |  |  |  |  |  |
| 17.1 | Top |  | Extension mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.2 | Bottom |  | Extension mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Shortening mm (shortening may not comply with EN115) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Balustrade |  | Vertical, Slimline , Tempered safety glass (10mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Balustrade profile |  | Hairline Stainless steel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Handrail |  | Standard |  |  |  |  | EHC (optional) |  |  |  |  |  |  | Semprit (optional) |  |  |
| 20.1 | Handrail color |  | Black |  |  |  |  | Colored, color No. (optional) |  |  |  |  |  |  |  |  |  |
| 21 | Inner & outer decking |  | Hairline Stainless steel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | Skirting panel |  | Hairline Stainless steel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | Panel of landing plate |  | Punched stainless steel, thickness 1mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Etched Stainless steel (optional, strip type, pattern No.RJ-151) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Extrude aluminum(optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | Logo on landing plate |  | Asia Schneider (Thailand) Co., Ltd. |  |  |  |  |  |  |  |  |  |  | Without |  |  |  |
| 25 | Logo on comb plate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | Pallet |  | One piece Die-cast aluminum, with yellow painting line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 | Combs |  | Die-cast aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Die-cast aluminum, yellow painting (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | Controller |  | PLC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 | Control type |  | Star-delta |  |  |  |  |  |  |  |  |  |  | VVVF (optional) |  |  |  |
| 29.1 | Moving type for Y-△ |  | Normal (only rated speed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Auto-start by diffuse sensor (optional, stop / rated speed, no lower speed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29.2 | Moving type for VVVF |  | Photocell on skirting (optional, lower / rated speed, no stop) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Auto-start by diffuse sensor (optional, stop / lower speed / rated speed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The width of trolley and its contents should be at least 400 mm less than pallet width, especially be cautious of pallet width of 800mm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ⅱ Safety features |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Stop button |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2 | Key switch |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 3 | Handrail entrance contact |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 4 | Broken pallet-chain contact |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5 | Pallet sag contact |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 6 | Phase monitor |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 7 | Zinc steel flying wheel cover |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 8 | Motor overload |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 9 | Motor overheat |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 10 | Comb contact |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 11 | Maintenance interlock |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 12 | Alarm buzzer |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 13 | Over-speed & unintended reversal monitor |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 14 | Pallet reversing fences |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 15 | Handrail anti-static roller |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 16 | Service brake release contact |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 17 | Pallet anti-static brush |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 18 | Earth leakage circuit-breaker for illumination circuit |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 19 | Soft stop |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 20 | Pallet missing contact |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 21 | Landing plate switch |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 22 | Alarm for stop distance overrun |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 23 | Handrail speed monitor |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 24 | Additional emergency stop device |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 25 | Power box |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 26 | Auto lubrication system (with oil collector) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 27 | LCD fault display (on the outer decking) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 28 | Fire interface (24Vdc/110Vac, NC dry contact and cable to be supplied by client) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| Ⅲ Optional function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Safety brake on main shaft |  |  |  | Yes, (required for rise exceed 6m) |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Drive-chain broken contact |  |  |  | Yes, (required for safety brake) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Machine room lighting |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 4 dry contacts for remote monitoring |  |  |  | Yes, up, down, malfunction, alarm |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Skirting lighting |  |  |  | Yes, LED dot type (can’t co-exist with two-rows skirting brush) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Yes, continues, LED lights with diffusor |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Pallet gap lighting |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Balustrade lighting |  |  |  | Yes, LED ring type |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Handrail broken contact |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Emergency stop device at the inclined section |  |  |  | Yes, required in case of L exceed 40m |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Machine room stair |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | LED comb lighting |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | LED traffic light (required in case of auto-start) |  |  |  | Each one at upper and bottom end |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Cable connector at truss intersection |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Outside cladding |  |  |  | Painted steel |  |  |  |  |  | Hairline stainless steel |  |  |  |  |  |  |
| 14.1 | Side(s) of cladding |  |  |  | Left |  |  |  |  |  | Right |  |  |  |  | Bottom |  |
| 14.2 | Cladding color for painted steel |  |  |  | Same as truss |  |  |  |  |  | Color No |  |  |  |  |  |  |
| 15 | Treatment of skirting panel surface: with TEFLON coating |  |  |  | Transparent |  |  |  |  |  | Black color |  |  |  |  |  |  |
| 16 | Skirting brush |  |  |  | Yes, single row |  |  |  |  |  |  |  |  |  |  |  |  |
| IV Optional tools |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Controller lifting device |  |  | One for per unit |  |  |  |  |  | for per order |  |  |  |  |  |  |  |
| 2 | Maintenance fence |  |  | One for per unit |  |  |  |  |  | for per order |  |  |  |  |  |  |  |
| 3 | Hoisting steel cable |  |  | One for per unit |  |  |  |  |  | for per order |  |  |  |  |  |  |  |
| 4* | Inspection handle |  |  | One for per unit |  |  |  |  |  | for per order |  |  |  |  |  |  |  |
| 5* | Portable lighting |  |  | One for per unit |  |  |  |  |  | for per order |  |  |  |  |  |  |  |
| Ⅴ Special item |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |