Technical Specification for New Escalator

| Buyer |  |  |  |  |  |  |  | Project Code |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contract No. |  |  |  |  |  |  |  | Project name |  |  |  |  |  |  |  |  |  |  |
| Installation site |  |  |  |  |  |  |  | Quantity |  |  |  |  |  |  |  |  |  |  |
| Delivery ex works |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Design reference: EN115 Note: If there is another standard used in local, please contact ASIA SCHNEIDER. The consulted result must be added to the contract. If the required specifications are not listed, please contact us. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I Main feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Inclination |  | 35° |  |  |  |  |  |  |  | 30° |  |  |  |  |  |  |  |
| 2 | Horizontal steps |  | Two |  |  |  |  |  |  |  | Three |  |  |  |  |  |  |  |
| 3 | Step width (mm) |  | 1000 |  |  |  |  |  | 800 |  |  |  |  |  |  | 600 |  |  |
| 4 | Vertical rise |  | H= mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Horizontal span |  | L= mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Intermediate support |  | No |  |  |  |  |  | One |  |  |  |  |  |  |  |  |  |
| 7 | Motor |  | kW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Power voltage |  | 380V |  |  |  | 400V |  |  |  | 415V |  |  |  |  |  |  | V |
| 9 | Lighting voltage |  | 220V |  |  |  | 230V |  |  |  | 240V |  |  |  |  |  |  | V |
| 10 | Frequency |  | 50Hz |  |  |  |  |  |  |  | 60Hz |  |  |  |  |  |  |  |
| 11 | Delivery in |  | Whole piece |  |  |  |  |  | Two pieces |  |  |  |  |  |  | Pieces |  |  |
| 12 | Arrangement |  | Single |  |  |  |  |  | Parallel |  |  |  |  |  |  | Continuous |  |  |
|  |  |  | Crisscross |  |  |  |  |  |  |  | According to layout |  |  |  |  |  |  |  |
| 13 | Standard installation |  | Yes |  |  |  |  |  | No ( If no, please fill 14.1 and 14.2 ) |  |  |  |  |  |  |  |  |  |
| 13.1 | Top |  | Extension mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.2 | Bottom |  | Extension mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Shortening mm (Shortening may not comply with EN115) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Handrail |  | Standard brand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | EHC (optional) |  |  |  |  |  |  | Choose one for outdoor installation |  |  |  |  |  |  |  |  |
|  |  |  | Semprit (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 | Handrail color |  | Black |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Colored, color No. (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Panel of landing plate |  | Punched stainless steel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Etched Stainless steel, strip type, pattern No. RJ-151 (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Extrude aluminum (optional, required for outdoor installation) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Logo on landing plate |  | ASIA SCHNEIDER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Logo on comb plate |  | ASIA SCHNEIDER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Combs |  | Die-cast aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Die-cast aluminum, yellow painting (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Step |  | Aluminum alloy Die-cast, gray color |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aluminum alloy Die-cast, gray color, yellow synthetic resin (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aluminum alloy Die-cast, gray color, yellow painting line (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Moving type |  | Y-△, normal (only rated speed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Y-△, auto-start by diffuse sensor (optional, stop / rated speed, no lower speed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | VVVF, photocell on skirting (optional, lower / rated speed, no stop) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | VVVF, auto-start by diffuse sensor (optional, stop / lower speed / rated speed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | Other parameters |  | 1. The default installation environment is indoor, the required features for Semi-outdoor and outdoor installation are listed in project V and VI; 2. Speed is 0.5 m/s; 3. The beam of truss is angle steel with normal painting (for indoor); 4. The gear box type is worm gear; 5. Vertical balustrade with tempered safety glass (10mm) and profile of hairline stainless steel; 6. The material of inner & outer decking and skirting panel is hairline stainless steel; 7. The controller type is . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| II Safety features |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Stop button |  |  |  | 2 |  |  |  | Key switch |  |  |  |  |  |  |  |  |  |
| 3 | Handrail entrance contact |  |  |  | 4 |  |  |  | Step-chain contact |  |  |  |  |  |  |  |  |  |
| 5 | Step sag contact |  |  |  | 6 |  |  |  | Phase monitor |  |  |  |  |  |  |  |  |  |
| 7 | Zinc steel flying wheel cover |  |  |  | 8 |  |  |  | Step reversing fences |  |  |  |  |  |  |  |  |  |
| 9 | Motor overload |  |  |  | 10 |  |  |  | Motor overheat |  |  |  |  |  |  |  |  |  |
| 11 | Comb contact |  |  |  | 12 |  |  |  | Maintenance interlock |  |  |  |  |  |  |  |  |  |
| 13 | Alarm buzzer |  |  |  | 14 |  |  |  | Overspeed & unintended reversal monitor |  |  |  |  |  |  |  |  |  |
| 15 | Handrail anti-static roller |  |  |  | 16 |  |  |  | Step anti-static brush |  |  |  |  |  |  |  |  |  |
| 17 | Service brake release contact |  |  |  | 18 |  |  |  | Soft stop |  |  |  |  |  |  |  |  |  |
| 19 | Handrail speed monitor |  |  |  | 20 |  |  |  | Power box |  |  |  |  |  |  |  |  |  |
| 21 | Landing plate switch |  |  |  | 22 |  |  |  | Alarm for stop distance overrun |  |  |  |  |  |  |  |  |  |
| 23 | Step missing contact |  |  |  | 24 |  |  |  | Auto lubrication system (with oil collector) |  |  |  |  |  |  |  |  |  |
| 25 | LCD fault display |  |  |  | 26 |  |  |  | Earth leakage protection for illumination circuit |  |  |  |  |  |  |  |  |  |
| 27 | Fire interface (24Vdc/110Vac, NC dry contact and cable to be supplied by client.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| III Optional function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Safety brake on main shaft |  |  |  |  | Yes, must required for rise exceed 6m |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Drive-chain broken contact |  |  |  |  | Yes, must required for safety brake |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 4 dry contacts for remote monitoring |  |  |  |  | Yes, up, down, malfunction, alarm |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | LED traffic light (required in case of auto-start) |  |  |  |  | Yes, Each one at upper and bottom end |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | LED comb lighting |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | LED step gap lighting |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Balustrade lighting |  |  |  |  | Yes, LED ring type |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Machine room lighting |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Skirting lighting |  |  |  |  | Yes, LED dot type (can’t co-exist with two-rows skirting brush) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Yes, LED lights with diffusor |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Skirting contact |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Zinc steel drive chain cover (required for outdoor installation) |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Step upthrust contact |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Handrail broken contact |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Machine room stair |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Cable protection |  |  |  |  | Metal raceway |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Plastic raceway |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Anti-climb device |  |  |  |  | Yes, |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Cable connector at truss intersection |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Outside cladding |  |  |  |  | Painted steel |  |  |  |  |  |  |  | Hairline stainless steel |  |  |  |  |
| 18.1 | Side(s) of cladding |  |  |  |  | Left |  |  |  |  |  | Right |  |  |  |  | Bottom |  |
| 18.2 | Cladding color (for painted steel) |  |  |  |  | Same as truss |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Treatment of skirting panel surface: with TEFLON coating |  |  |  |  | Transparent |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Black color |  |  |  |  |  |  |  |  |  |  |  |  |
| 20* | Skirting brush |  |  |  |  | Yes, single row |  |  |  |  |  |  |  |  |  |  |  |  |
| * Options No. 20 is compulsory for EN115:1-2008 or EN115:1-2008+A1:2010. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IV Optional tools： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Controller lifting device |  |  |  | One for per unit |  |  |  |  |  |  |  | for per order |  |  |  |  |  |
| 2 | Maintenance fence |  |  |  | One for per unit |  |  |  |  |  |  |  | for per order |  |  |  |  |  |
| 3 | Hoisting steel cable |  |  |  | One for per unit |  |  |  |  |  |  |  | for per order |  |  |  |  |  |
| 4* | Inspection handle |  |  |  | One for per unit |  |  |  |  |  |  |  | for per order |  |  |  |  |  |
| 5* | Portable lighting |  |  |  | One for per unit |  |  |  |  |  |  |  | for per order |  |  |  |  |  |
| * At least one inspection handle and one portable lighting for per order are required if comply with EN115:1-2008 or EN115:1-2008+A1:2010. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V Semi-outdoor installation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Treatment of truss |  |  | 3 layers painting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Step |  |  | Aluminum alloy Die-cast |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Protection rate |  |  | Control box: IP54; inverter (if selected): IP54; safety switch: IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Cooling fan in control box |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Cooling fan in machine room |  |  | Recommended if ambient temperature over 40℃ or upper machine room direct expose to sunlight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Float switch at lower machine room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Comb heater |  |  | Required for ambient temperature below 4℃ and outdoor machine room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Truss heater |  |  | Required for ambient temperature below 0℃ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VI Outdoor installation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Stainless steel material |  |  |  | 304# |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Treatment of truss |  |  |  | Hot dip galvanized |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Step |  |  |  | Aluminum alloy Die-cast |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Panel of landing plate |  |  |  | Extrude aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Float switch at lower machine room |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Oil-water separator |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Zinc steel drive chain cover |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Chain protection: zinced |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Protection rate |  |  |  | Control box: IP54; inverter (if selected): IP54; safety switch: IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Insulation rate |  |  |  | Motor: level F; electromagnet: level E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Cooling fan in control box |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Cable protection |  |  |  | Plastic raceway |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Steel raceway |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Handrail |  |  |  | EHC (NT3000) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Semprit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Cooling fan in machine room |  |  |  | Recommended if ambient temperature over 40℃ or upper machine room direct expose to sunlight |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Comb heater |  |  |  | Required for ambient temperature below 4℃ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Truss heater |  |  |  | Required for ambient temperature below 0℃ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VII Special item： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |